MOTO-INDICATOR Smart Bluetooth Turn-Signal System Integrated with Google Maps Navigation ────────────── Product Proposal Document Version 1.0 ¦ 2025

# 1. Executive Summary

Moto-Indicator is a software-first, Bluetooth-connected smart indicator system for motorcycles. It automatically triggers the bike's left or right turn indicator during navigation, responding to GPS turn data from a navigation app, and plays a distinct audible cue — so the rider never needs to take their eyes off the road or their hands off the handlebar.

The system works by pairing the motorcycle's indicator relay module with the rider's smartphone via Bluetooth Low Energy (BLE). When the navigation app detects an upcoming turn within the speed-adaptive trigger zone, it sends a "L" (Left) or "R" (Right) command to the paired device named "indicator", which activates the correct blinker and plays a side-specific audio cue.

| CORE GOAL | Hands-free, eyes-free indicator control via software — no button press required while riding. |
| --- | --- |

# 2. Software Architecture

## 2.1  System Components (Software Only)

| Component | Platform | Role |
| --- | --- | --- |
| Navigation App Layer | Android / iOS | GPS tracking, turn detection, speed-adaptive trigger distance, BLE command sender |
| Bluetooth BLE Stack | OS-level (Android BLE / CoreBluetooth iOS) | Device scan, pairing handshake, GATT write to "indicator" device |
| Command Protocol | Custom BLE GATT Characteristic | Sends L / R / C (cancel) as single-byte UTF-8 over BLE notify |
| Audio Cue Engine | App-side TTS or tone player | Left: short-short beep (di-di). Right: long-short beep (daa-di). |
| Auto-Cancel Logic | App-side GPS heading comparator | Detects heading delta >65 degrees OR 7-second timeout; sends C command |

## 2.2  Communication Protocol

The BLE command set is minimal by design — three single-byte ASCII commands:

| Command | Meaning | Triggered by |
| --- | --- | --- |
| L | Activate left indicator + left audio cue | Navigation app detects left turn within trigger distance |
| R | Activate right indicator + right audio cue | Navigation app detects right turn within trigger distance |
| C | Cancel / turn off both indicators | GPS heading change OR 7-second auto-timeout OR manual override |

## 2.3  Speed-Adaptive Trigger Distance

The trigger distance adjusts dynamically based on GPS-reported speed so the indicator fires early enough at highway speeds:

| Speed | Trigger Distance | Reaction Time Given |
| --- | --- | --- |
| Below 20 km/h | 20 m | 3.6 sec |
| 20 – 40 km/h | 35 m | 3.5 – 6.3 sec |
| 40 – 60 km/h | 55 m | 3.3 – 4.9 sec |
| 60 – 80 km/h | 75 m | 3.3 – 4.5 sec |
| Above 80 km/h | 100 m | 4.5 sec |

# 3. Integration Approaches

There are three realistic ways to integrate this feature into the software ecosystem. Each is analysed below for feasibility, difficulty, and recommended path.

## Option A — Standalone Companion App

| What it is | A dedicated app (Android/iOS) that runs alongside Google Maps, reads navigation state via Waze/Google Maps deep links or the platform direction API, manages BLE pairing, and sends commands independently. |
| --- | --- |
| Practical ease | HIGH — Fully within developer control. Publishable on Play Store / App Store. No partnership required. Can be built today. |
| Drawback | Requires the user to run two apps simultaneously. Turn detection accuracy depends on the availability of turn-by-turn data from the background navigation app, which Google Maps does not expose via public API. |

## Option B — Google Maps Platform SDK (Recommended)

| What it is | Build a custom navigation app using the Google Maps Navigation SDK (available to fleet/mobility partners). The app embeds real navigation with full turn-by-turn step access and adds the BLE indicator layer on top. |
| --- | --- |
| Practical ease | MEDIUM — Requires Google Maps Navigation SDK access (apply at developers.google.com/maps/documentation/navigation). SDK provides accurate maneuver step data (TURN_LEFT, TURN_RIGHT, etc.) that feeds directly into BLE trigger logic. |
| Advantage | Single app. Accurate maneuver detection. Full map UI. This is the recommended path — it is the closest to "Google Maps with the feature built in." |

## Option C — Google Maps Waze Carpool / OEM Integration

| What it is | A formal partnership with Google to ship this feature inside the consumer Google Maps app. This would require Google to expose a BLE/device API within Maps and approve the feature for the app. |
| --- | --- |
| Practical ease | VERY LOW — This requires a formal OEM/partner agreement, is a multi-year process, and is not within scope for an independent developer. Not a viable near-term path. |

# 4. Practical Difficulties

## 4.1  Technical Difficulties

- GPS accuracy in dense urban environments (signal bounce off buildings) can cause 5–15 m position errors, potentially triggering the indicator slightly early or late.

- Turn-by-turn data access: Consumer Google Maps does not expose navigation step data to third-party apps. The Navigation SDK (Option B) solves this but requires SDK approval.

- BLE connection reliability: BLE drops when the phone screen locks on some Android versions. The app must request BLUETOOTH_CONNECT and run a foreground service with a persistent notification to maintain the connection.

- Auto-cancel accuracy: Heading change detection works well at speeds above 15 km/h but can misfire in tight parking lots or U-turns. A minimum speed threshold of 10 km/h for auto-cancel is recommended.

- Audio cue delivery: If the rider uses Bluetooth earphones paired to the phone, the audio must be routed correctly to the earphones and not the phone speaker. This requires the app to use AudioManager.STREAM_MUSIC with a Bluetooth SCO or A2DP profile.

## 4.2  Platform / OS Difficulties

- iOS background BLE: iOS restricts background Bluetooth usage. The app must declare "bluetooth-peripheral" in the UIBackgroundModes plist key and may still be suspended by the OS after ~3 minutes. A workaround is to use the Core Location "significant change" API to keep the app alive.

- Android battery optimisation: Aggressive battery savers (OEM-specific on Xiaomi, Samsung, Oppo) kill background BLE services. The app must prompt the user to whitelist it from battery optimisation.

- Permissions: The app requires BLUETOOTH_SCAN, BLUETOOTH_CONNECT, ACCESS_FINE_LOCATION, and FOREGROUND_SERVICE permissions. Android 12+ requires runtime permission requests for BLE access.

## 4.3  Regulatory / Safety Difficulties

- In most jurisdictions, the physical indicator circuit of a vehicle is a safety-critical component. Any modification to it must comply with motor vehicle regulations (e.g., CMVR in India, ECE in Europe). A purely software-driven relay is in a grey area — legal review is advised before commercialising.

- Liability: If the indicator fires incorrectly during navigation (e.g., wrong side due to GPS error), it could cause an accident. The system must be designed with the manual override as the primary and legal indicator control.

# 5. Recommended Approach

| VERDICT | Build a standalone navigation app using the Google Maps Navigation SDK (Option B). This is the most practically achievable path within 6–12 months. |
| --- | --- |

## 5.1  Phased Development Plan

- Phase 1 (Months 1–2): Proof of concept — standalone app with mock GPS data, BLE scan/pair to "indicator" device, correct L/R/C command flow, and dual audio cues.

- Phase 2 (Months 3–4): Apply for Google Maps Navigation SDK access. Integrate real turn-by-turn maneuver data. Replace mock GPS with live data.

- Phase 3 (Months 5–6): Speed-adaptive trigger, auto-cancel with heading detection, iOS background BLE support, battery-optimisation whitelist prompt.

- Phase 4 (Months 7–8): User testing with actual riders, refinement of trigger distances, safety audit, soft launch on Play Store as closed beta.

## 5.2  Technology Stack

| Layer | Recommended Technology |
| --- | --- |
| Android App | Kotlin + Jetpack Compose + Google Maps Navigation SDK |
| iOS App | Swift + SwiftUI + Google Maps Navigation SDK (iOS) + CoreBluetooth |
| BLE Stack | Android BLE API / CoreBluetooth — GATT client writing to characteristic UUID |
| Navigation Data | NavSDK StepInfo.Maneuver enum (TURN_LEFT, TURN_RIGHT, etc.) |
| Audio Cues | Android AudioTrack / iOS AVAudioPlayer with pre-recorded tone files |
| Background Service | Android: ForegroundService with wake-lock. iOS: BGTaskScheduler + BTLE background mode |

# 6. Safety Features (Non-Negotiable)

- Manual override always works: The physical handlebar indicator switch must override and cancel any auto-triggered indicator at any time.

- Fail-safe on disconnect: If BLE drops, the system must not trigger anything. The bike reverts to fully manual indicator operation.

- No haptic distraction: Audio cues use the earphone/helmet speaker only. No phone vibration while riding.

- Indicator auto-cancel: Maximum 8 seconds blink duration regardless of navigation state. This prevents indefinite blinking if GPS auto-cancel fails.

- Speed gate: The system only operates above 5 km/h. At standstill or slow parking, auto-trigger is suppressed.

- Clear pairing name: The BLE device must advertise as "indicator" so the rider knows exactly what they are connecting to. No generic device names.

# 7. Option Comparison Summary

| Criteria | Option A Standalone App | Option B Nav SDK App | Option C Inside Google Maps |
| --- | --- | --- | --- |
| Developer control | Full | Full | None |
| Turn data accuracy | Medium (GPS only) | High (SDK maneuver) | Highest |
| BLE integration | Yes | Yes | Not available |
| Time to build (MVP) | 2–3 months | 4–6 months | 2+ years |
| Google partnership needed | No | SDK access only | Yes (full partner) |
| App Store publishable | Yes | Yes | No |
| Recommended | Fallback | ✓ Yes | ✗ No |

Moto-Indicator  |  Product Proposal v1.0  |  Confidential

This document is intended for engineering and product teams. Redistribution requires written consent.