NestGuard System Architecture & Vendor Engineering Specification

Document Version: 1.0

Target Release: Enterprise / IoMT Ready

Subject: Hardware-Software Mapping, Edge AI Pipelines, and System Deployment Matrix

1. High-Level System Architecture

┌─────────────────────────────────────────────────────────────────────────┐

│                            HARDWARE LAYER                               │

│  • Physical Sensors (IR Thermal, mmWave Radar, Gas, Camera, Mic, etc.) │

└────────────────────────────────────┬────────────────────────────────────┘

│ (Hardware Signals / Busses)

┌────────────────────────────────────▼────────────────────────────────────┐

│                  EDGE FIRMWARE & ON-CHIP SOFTWARE                       │

│  • ESP32-S3 Firmware (FreeRTOS / C++)                                   │

│  • Edge AI Co-Processor (Edge Impulse / TensorFlow Lite Micro)          │

└────────────────────────────────────┬────────────────────────────────────┘

│ (MQTT / WebSockets over Wi-Fi)

┌────────────────────────────────────▼────────────────────────────────────┐

│                     APPLICATION & DASHBOARD LAYER                       │

│  • Caregiver Mobile App / Dashboard (React Native or Flutter / HTML)    │

│  • Cloud/Backend Broker (Node.js/Python, AWS IoT Core or HiveMQ)        │

└─────────────────────────────────────────────────────────────────────────┘

2. Technical Node Mapping & Deployment Specifications

Section 1: Physical Sensors & Transducers (Hardware Core)

- IR Thermal Array (MLX90640)

  - Hardware: 32×24 Far-Infrared Sensor Array.

  - Software Stack: Adafruit_MLX90640 C++ Driver / Linux i2c-dev library.

  - Target Deployment: Compiles directly into the primary ESP32-S3 firmware binary.

- 60GHz mmWave Radar (BGT60TR13C)

  - Hardware: Radar SoC IC mounted on overhead housing.

  - Software Stack: Radar MCU SDK / C-based UART frame parser.

  - Target Deployment: Executes as a dedicated real-time task on the FreeRTOS scheduler.

- Gas & Environmental Sensor (BME688 / MQ Series)

  - Hardware: Metal-oxide gas sensor for VOC and NH3 detection.

  - Software Stack: Bosch BSEC (Bosch Sensortec Environmental Cluster) library.

  - Target Deployment: Compiled as a static binary library into the ESP32 firmware.

- Ambient Environment Sensor (BME280 / DHT22)

  - Hardware: Temperature and relative humidity module.

  - Software Stack: Adafruit_BME280 or C-driver wrapper.

  - Target Deployment: ESP32 firmware task executing on a 1 Hz polling loop.

- Mattress Load Cells (HX711) & 6-Axis IMU (MPU6050)

  - Hardware: 4× corner strain gauge load cells with HX711 ADC + 6-DOF IMU under mattress.

  - Software Stack: HX711-multi C++ library & MPU6050_light driver.

  - Target Deployment: ESP32 firmware running a continuous weight-fusion algorithm.

- RGB / NIR Camera Module (OV2640 / OV5640)

  - Hardware: Camera module with IR cut filter connected via DVP or MIPI-CSI.

  - Software Stack: esp32-camera component / Linux V4L2 drivers.

  - Target Deployment: Edge AI Co-Processor or Linux Micro-SBC interface.

Section 2: Communication Protocols & Edge Models

- Thermal Processing Unit (Heat-Grid Calibration Model)

  - Software Stack: C++ Matrix math library / NumPy for linear interpolation.

  - Target Deployment: Executes on the Edge Hub to upscale raw 32×24 frames into a 128×96 thermal overlay matrix.

- On-Chip Phase Classifier (1D-CNN Time-Series Model)

  - Software Stack: TensorFlow Lite for Microcontrollers (TFLM) / Edge Impulse C++ SDK.

  - Target Deployment: Flashed into internal flash memory of the ESP32-S3 or Radar DSP.

- Audio Analysis Loop (Mel-Spectrogram CNN)

  - Software Stack: CMSIS-DSP (real-time FFT) + Quantized TFLM Audio Classifier.

  - Target Deployment: Runs directly on the ESP32-S3 DMA Audio Pipeline via I2S.

- Bus Protocols & Low-Level Drivers

  - Software Stack: FreeRTOS drivers (i2c_master, i2s_driver, uart_driver).

  - Target Deployment: Low-level board support package (BSP) compiled into .bin firmware.

- Stillness & Weight Fusion Gate (Load + IMU)

  - Software Stack: Custom C++ Kalman Filter / Moving Average algorithm.

  - Target Deployment: ESP32 firmware task maintaining a 30-second verification window.

Section 3: Central Hub & Edge AI Vision Core

- Vision Core (MobileNet Face & Obstruction Tracking)

  - Software Stack: OpenCV + TensorFlow Lite Micro / MobileNetV2 SSD runtime.

  - Target Deployment: Runs on dedicated Edge AI Hardware (ESP32-S3 vector core or Linux Micro-SBC like K210/Jetson).

- ESP32 Central Processing Hub (Event Aggregator & Local Alert Logic)

  - Software Stack: C++ / FreeRTOS, esp-mqtt, esp-websocket, cJSON.

  - Target Deployment: Main MCU flash memory; acts as the primary local event aggregator and alert orchestrator.

Section 4: Caregiver Interface & Cloud Gateways

- Backend Communication Gateway

  - Software Stack: MQTT Broker (EMQX or AWS IoT Core) + Node.js / Python FastAPI server.

  - Target Deployment: Cloud Container (Docker / AWS ECS / DigitalOcean) or local edge gateway.

- Caregiver Application / Executive Dashboard

  - Software Stack: React Native / Flutter (Mobile) or HTML5 / Tailwind CSS / WebSockets (Web).

  - Target Deployment: iOS App Store / Google Play Store / Hosted Cloud CDN (Vercel / Netlify).

3. Summary Deployment Matrix for Vendor Delivery

| Functional Node | Target Hardware / IC | Software Framework | Delivery Artifact |
| --- | --- | --- | --- |
| Sensors & Transducers | MLX90640, BME688, HX711, INMP441, MPU6050 | C++ Hardware Drivers / BSP | Firmware .bin file flashed on MCU |
| Edge Micro-Models | ESP32-S3 / Edge Processing Hub | TensorFlow Lite Micro / CMSIS-DSP | Quantized .tflite model embedded in C++ headers |
| Vision Core | Camera Module + AI Coprocessor | OpenCV + MobileNetV2 | Executable binary / C++ container on Edge SoC |
| Central Processing Hub | ESP32-S3 Board | FreeRTOS / ESP-IDF C++ | Firmware main binary (main.bin) |
| Caregiver Interface | Smartphone / PC Web Browser | React Native / HTML5 + WebSockets | .apk / .ipa mobile packages or Hosted Web App |